MARYLAND DEPARTMENT OF NATURAL RESOURCES

RESOURCE ASSESSMENT SERVICE

MARYLAND GEOLOGICAL SURVEY

Abbreviated Metadata

Hydrographic data in Microsoft Excel format

Triadelphia Reservoir

2015

Identification Information

Description: This data set consists of a Microsoft Excel (.xlsx) data file, containing depth soundings collected in Triadelphia Reservoir.  The sounding data were collected by the Maryland Geological Survey from April 20, 2015 to May 1, 2015.

Purpose:  In response to a request by Washington Suburban Sanitary Commission (WSSC), Maryland Geological Survey (MGS) was contracted to study the bathymetry and sedimentation of Triadelphia Reservoir located in Howard and Montgomery Counties in the State of Maryland.  The objectives for this study were to 1) determine remaining storage capacity of Triadelphia Reservoir; 2) determine sediment accumulation in Triadelphia Reservoir since the previous survey in 2004; 3) collect side scan sonar imagery and create a mosaic raster map for Triadelphia Reservoir.

Process:

Soundings- Track lines running perpendicular to the river channels were established for bathymetric surveying.  These track lines were spaced 50 meters apart and extended shoreline to shoreline.  Tie-in lines were run perpendicularly to the survey lines and along the axial channels of the reservoirs.  Additionally, survey track lines were run along the perimeter of the reservoirs to assist in the surface modeling analysis.  The bathymetry surveys were conducted in April of 2015 for Triadelphia Reservoir.

Bathymetric data were collected using a Thales Navigation (Ashtech) DGPS (model DG-16; L1 code and carrier with SBAS and differential beacon corrections; 5Hz Update rate) and a Knudsen 320B/P dual frequency echosounder with sounding frequencies of 200 kHz and 28 kHz.  The echosounder transducer is a KEL771 dual frequency transducer with a 200 kHz beam angle of 4 degrees and a 28 kHz beam angle of 29 degrees.  The echosounder generated acoustic pulses for bottom recognition at a rate of 2 Hz.  The pulse width was set to automatically change between 0.2 mS and 0.8 mS depending on the depth of the water.  The transmitted acoustic wave reflected off the density gradient separating the water column from the bottom sediment.  The returned acoustic wave is received by the transducer, and the time separation between the sent and the returned wave is recorded.  This time separation is directly proportional to distance.  The recordings were then filtered for points that were outside of the gate window (2 meters) and integrated within the echosounder to produce an accurate measurement from the transducer to the water/sediment interface.  The DGPS position and Knudsen depth data were integrated real time using HYPACK software. At an average vessel speed of 4 knots, a depth sounding was collected approximately every 1.0 m [3.3 ft] along the survey track-lines.  This data was stored along with the GPS location and positional latency in a laptop computer.  Navigation was provided using the Thales Navigation DGPS integrated with the HYPACK software and HYPACK planned lines.  DGPS differential corrections broadcast by the United States Coast Guard provided a real-time horizontal accuracy of +/- 1 meter [3.3 feet] using the Annapolis and Hagerstown DGPS sites.  The Thales Navigation (Ashtech) DGPS and the echosounder were checked against known horizontal and vertical measurements during the survey.  The echosounder was also calibrated throughout the depth range of the reservoir during the study period.

Mean Pool Level Adjustment - The bathymetric data collected presented measurements based upon the distance between the surface of the water in the reservoir and the top of the water-sediment interface.  Due to fluctuations in the reservoirs level, the bathymetric data was adjusted to a known reference level using water level measurements recorded by a gauge operated by WSSC.  MGS also installed a water level gauge during data collection as a reference to the recorded WSSC gauge recordings. This survey data was adjusted to the Mean Pool Level (MPL) of 366.4 feet above Mean Sea Level (MSL) for Triadelphia Reservoir.

Accuracy: The accuracy of the post-processed bathymetric data is ± (0.1 ft + 1% of the water depth) to MPL.  The accuracy of the horizontal DGPS data is ±1.0 m [±3.3 ft].

Attribute definitions:  Triadelphia2015.xlsx consists of 3 columns by 427,603 rows.  The first row contains the header information described in Table 1.

| Table 1. Column Headings and definitions for Microsoft Excel sounding data for Triadelphia Reservoir. |  |  |
| --- | --- | --- |
| Column | Heading | Definition/explanation |
| 1 | Northing | Northing coordinate (Universal Transverse Mercator grid, zone 18, NAD83), in meters. |
| 2 | Easting | Easting coordinate (Universal Transverse Mercator grid, zone 18, NAD83), in meters. |
| 3 | MPL_depth | Final adjusted depth in meters, depths are below Mean Pool Level of the reservoir; adjustments include corrections for speed of sound and offset of transducer; mean pool level adjustments were derived from water level data collected at the time the sounding were recorded. Mean Pool Level (MPL) for Triadelphia is 111.68 meters (366.4 feet) above Mean Sea Level (MSL) and referenced to NGVD29. |

Point of Contact:

Contact Person Primary

Stephen Van Ryswick

Maryland Geological Survey

2300 St. Paul Street

Baltimore, Maryland  21218-5210

Phone: (410) 554-5544

Fax: (410) 554-5502

Email: stephen.vanryswick@maryland.gov

Use Constraints:  Acknowledgment of the Maryland Geological Survey, Resource Assessment Service, MD Department of Natural Resources, shall be made in products derived from these data.